How to enable Wi-Fi/ Bluetooth with

EWM-W177M201E

Prerequisite

- MIO-5373 with Ubuntu Desktop 22.04.2 LTS Linux 5.17.15 installed.

Check Hardware Status

- Check the presence of Wi-Fi hardware

# lspci

- Check the presence of Bluetooth hardware

# lsusb

Test

After boot up, please login user account on Ubuntu, and refer following instructions to test module.

- Test by GUI

  - Wi-Fi

- Select the option Wi-Fi

- Select the SSID of AP

- Input your password of AP

- Open the terminal to check interface and network connection

  - Check interface and you will see the interface as “wlp2sX”

# ifconfig

  - Check network connection after obtained IP address

# ping –c 3 8.8.8.8

1.2 Bluetooth

- Select the option Bluetooth

- You will see the result in scanning as below list

- Test by command line

  - Wi-Fi

- Stop some services

# sudo systemctl stop wpa_supplicant

# sudo systemctl stop NetworkManager

- Set Wi-Fi client configuration with /etc/wpa_supplicant.conf

E.g. Follows is set with AP with SSID as “TEST” and pre-shared key as

“123456789” to connect to Wi-Fi 4 (802.11n) or Wi-Fi 5 (802.11ac) AP

ctrl_interface=/var/run/wpa_supplicant

ctrl_interface_group=0

update_config=1

network={

ssid="TEST"

psk="123456789"

}

Or, refer to follows to connect to Wi-Fi 6 AP

ctrl_interface=/var/run/wpa_supplicant/

update_config=1

pmf=1

sae_pwe=1

network={

ssid="TEST"

proto=RSN

key_mgmt=SAE

ieee80211w=2

group_mgmt=AES-128-CMAC

psk="123456789"

}

- Enable Wi-Fi and check interface

# sudo rfkill unblock wifi

# sudo iwconfig

Note: you will see the interface like wlp2sX, e.g. wlp2s0

- Running wpa_supplicant

# sudo wpa_supplicant -s -B -i wlp2s0 -c /etc/wpa_supplicant.conf

- Get IP address by DHCP and check network connection

# sudo dhclient wlp2s0

# ping -c 3 8.8.8.8

  - Bluetooth

- Enable Bluetooth

# sudo rfkill unblock bluetooth

- Check Bluetooth interface

# hciconfig -a

Note: you will see the interface like hciX, e.g. hci0

- Turn on/off scan

# sudo bluetoothctl scan on