AIW/EWM FAQ

| Category | AIW/EWM | Date | 2022/01/25 |
| --- | --- | --- | --- |
| Keyword | uAP (Software AP) ,AIW-154, Ubuntu 20.04 LTS,Linux kernel 5.8.0-53,X86,ARM platform support |  |  |

- Question:

How to setup software AP function when running AIW-154 on ubuntu linux OS?

- Instructions :

- Download whole AIW-154 software package by attached file and decompress on your desktop

https://advantecho365-my.sharepoint.com/:u:/g/personal/tim_huang_advantech_com/EWskDIWGfwhDoLXCNfGmQJYB12MQ_QLn_CqGU__l5cXgPw?e=piSDVo

- Rename original inbox driver to avoid activating when system boots up

(/lib/modules/5.8.0-53-generic/kernel/drivers/net/wireless/marvell/mwifiex)

Note:rename the filename from “mwifiex_pcie.ko” to “mwifiex_pcie_original.ko”

- Copy “nxp” whole folder under “/lib/firmware/”directory

- Compile driver package to generate ko files by “make”

NOTE: If your system is ARM base, please remember to modify the makefile before driver compiling

- Remark the x86 related configuration to switch for ARM base system

ARCH ?= arm64

#ARCH ?= x86

#KERNELVERSION_X86 :=    $(shell uname -r)

#KERNELDIR ?= /lib/modules/$(KERNELVERSION_X86)/build

<For X86 platform>

- Make sure both of driver ko files generate success in your directory

- Insert driver and active software ap function by following command

- insmod  mlan.ko

- insmod moal.ko mod_para=nxp/wifi_mod_para.conf drvdbg=0x7 cal_data_cfg=none

- Check these interfaces appear by “ifconfig”

- Create a new folder and fill in your ip table rules

- mkdir /etc/iptables/

- iptables -t nat -A POSTROUTING -o eth0 -j MASQUERADE

- iptables -A FORWARD -i eth0 -o uap0 -m state --state RELATED,ESTABLISHED -j ACCEPT

- iptables -A FORWARD -i uap0 -o eth0 -j ACCEPT

- iptables-save > /etc/iptables/iptables.rules

Notice: the eth0 is my interface name of ethernet and ensure the name is correct before filling

- Create iptables service in specific directory (/lib/systemd/system/iptables.service) and enable the iptable process by commands

<service_example>

[Unit]

Description=IPv4 Packet Filtering Framework

Before=network-pre.target

Wants=network-pre.target

[Service]

Type=oneshot

ExecStart=/usr/sbin/iptables-restore /etc/iptables/iptables.rules

ExecReload=/usr/sbin/iptables-restore /etc/iptables/iptables.rules

RemainAfterExit=yes

[Install]

WantedBy=multi-user.target

- systemctl –system daemon-reload

- systemctl enable iptables

- Enable network packet forwarding setting on specific file (/etc/sysctl.conf) and save it

- Install related software package(hostapd, udhcpc) and copy hostapd.conf & udhcpd.conf into “/etc/” directory

- sudo apt-get install hostpad

- sudo apt-get install udhcpd

<Example_udhcpd.conf>

<Example:hostapd.conf>

- Activate udhcpd and check if the process is working under system background

- udhcpd /etc/udhcpd.conf &

- ps aux | grep udhcpd

- assign ip to your software ap (uap0)

- ifconfig uap0 192.168.8.1 netmask 255.255.255.0 up

- Active hostpad to start up your AP

- Hostapd hostapd.conf &

- Try to connect your hotspot from other device and you will see the connection is established by following prompt message