How to enable WiFi/Bluetooth with AIW-162

Prerequisite

- ROM-5880 (SOM-DB2510) with Debian 10.13 (Linux kernel v4.19.232) on it

- Stop system background daemons by following commands with console utility.

#systemctl stop NetworkManager

#killall wpa_supplicant

- Extract “AIW-162_FW.zip” and copy files to one USB stick and plugged into any one USB port of SOM-DB251 platform and follows WiFi/ Bluetooth test steps next.

- AIW-162_FW.zip is including

- WiFi driver and firmware files:

bcmdhd.ko,

fw_bcm43752a2_pcie_ag.bin,

nvram_ap6275p_mp.txt,

clm_bcm43752a2_pcie_ag.blob

2) Bluetooth firmware: BCM4362A2_001.003.006.1059.1089.hcd

.

Test WiFi

- Bring up WiFi driver :

- Bring up WiFi driver with bcmdhd.ko if mount USB stick underlying /run/media/sda1

# cp /run/media/sda1/bcmdhd.ko /system/lib/modules/

# cp /run/media/sda1/fw_bcm43752a2_pcie_ag.bin

/vendor/etc/firmware/

# cp /run/media/sda1/nvram_ap6275p_mp.txt

/vendor/etc/firmware/nvram.txt

# cp /run/media/sda1/clm_bcm43752a2_pcie_ag.blob

/vendor/etc/firmware/

# insmod /system/lib/modules/ bcmdhd.ko

Note: you will see the interface wlan0 by “ifconfig –a” command

# ifconfig wlan0 up

- Test with WiFi AP

- Using editor (such as Vim) to create WiFi client configuration in /etc/wpa_supplicant.conf

(Following is one example as one AP named as “TEST” with pre-shared key

“123456789” for your reference)

# cat /etc/wpa_supplicant.conf

ctrl_interface=/var/run/wpa_supplicant

ctrl_interface_group=0

update_config=1

network={

ssid="TEST"

psk="123456789"

}

# wpa_supplicant -i wlan0 -c /etc/wpa_supplicant.conf -B

# dhclient wlan0

- If AP is configured with internet connection, user can use ping to check the internet availability as follows.

  - Check network connection after obtained IP address

# ping 8.8.8.8

Test Bluetooth

- Bring up Bluetooth Interface

- Bring up Bluetooth driver with follows if have mount USB stick underlying /run/media/sda1

# cp /run/media/sda1/BCM4362A2_001.003.006.1059.1089.hcd

/vendor/etc/firmware/BCM4362A2.hcd

# brcm_patchram_plus1 -d --enable_hci --no2bytes --tosleep 200000 –

baudrate 1500000 --patchram /vendor/etc/firmware/BCM4362A2.hcd

/dev/ttyS4 &

[ Notes ]

- Please wait seconds for “brcm_patchram_plus1” commane to finish

- If you see following console snapshot which means the patch is done

successfully , and then the interface “hci0” is ready by

checking with “hciconfig –a” command.

- Test Bluetooth Interface

- Using Bluetooth with following commands

# hciconfig hci0 up

# bluetoothctl

[bluetooth]# scan on

[bluetooth]# scan off

If there are any Bluetooth devices nearby you will see some devices listed by “devices” command as follows.

[bluetooth]# devices