EWM FAQ

| Category | EWM | Date | 2021/04/22 |
| --- | --- | --- | --- |
| Keyword | AIW-344FQ(EWM-C148FL02E),windows 10 OS (Version1809),UTX-3117 |  |  |

- Question:

How to establish LTE cellular network with AIW-344FQ(EWM-C148FL02E) on windows 10 OS x86 platform?

- Instructions :

Step 1

Insert AIW-344FQ(EWM-C148FL02E) module into mini PCI-E slot and both of Main and Aux has connected antenna cables already

Step 2

Download windows driver package by following URL and install on your OS

https://1drv.ms/u/s!AvzLe5sr2qg2g8kKnlENkfCyqiOLRg?e=szaQdI

Step 3

Enter to Setting section and enable cellular mode for establishing cellular connection

Step 4

With specific sim card, it must enter APN setting, username and password in advance

- Click advance option to add APN profile

- Add related configuration and apply it

Step 5

After enabling, the cellular function will auto-search 3G/4G signal around your place to establish connection

Step 6

Access the internet by browser and ensure if the webpage can show normal or not