EWM FAQ

| Category | EWM | Date | 2019/11/19 |
| --- | --- | --- | --- |
| Keyword | EWM-C117FL02E ,4G/LTE Connection |  |  |

- Question:

How to connect 4G/LTE with EWM-C117FL02E in RNDIS mode?

- Instructions :

Step 1

Download related utilities and driver from following link and decompress the file at local device

https://1drv.ms/u/s!AmPTlw75mrwLgzICSmT1IaNxFiEI

Step 2

Install cellular module on your device and make sure RNDIS driver has installed successfully

Step 3

Enter to device manager and ensure the u-blox USB modem interface has generated

Step 4

Execute m-center utility to configure related setting

Step 5

After selecting Com port, press connect button for connecting, then press “Get info” button to acquire modem information

Step 6

Input SIM card’s PIN Code

<Note>

Disable your sim card’s pin code to avoid inputting every time when you logon system

Step 7

Switch to AT terminal mode and select specific AT group (Set_APN) and input Commands orderly.

Step 8

If your SIM card already include username and password, please select “Set_APN_user/pwd” AT group and change APN/USERNAME/PASSWORD before sending commands.

Step 9

Close AT terminal session and exit m-center utility, then reboot system for establishing RNDIS connection automatically.

Step 10

After above configurations, you will see the connection is established to browse internet as usual.