How to enable WiFi/Bluetooth with AIW-169

Prerequisite

- RSB-4810 with Debian 10.12 (Linux kernel v4.19.232) on it

- Stop system background daemons by following commands with console utility.

# systemctl stop NetworkManager

# killall wpa_supplicant

- Extract “AIW-169_RSB4810_FW.zip” and copy files to one USB stick and plugged into any one USB port of RSB-4810 platform and follows WiFi/ Bluetooth test steps next.

“AIW-169_RSB4810_FW.zip” includes following files:

- WiFi driver files:

- rtkm.ko

- 8852ce.ko

- Bluetooth driver and firmware files:

- rtk_btusb.ko

- rtl8852cu_config

- rtl8852cu_fw

[* Notes ] WiFi 6GHz is only supported if RSB-4810 kernel 5.4 or above

Test WiFi

- Bring up WiFi driver :

- Bring up WiFi driver with 8852ce.ko if mount USB stick underlying /run/media/sda1 on RSB-4810

# cp /run/media/sda1/*.ko /tmp

# insmod rtkm.ko

# insmod 8852ce.ko

Note: you will see the interface wlan0 by “ifconfig –a” command

# ifconfig wlan0 up

- Test with WiFi AP

- Using editor (such as Vim) to create WiFi client configuration in /etc/wpa_supplicant.conf

(Following is one example as one AP named as “TEST” with pre-shared key

“123456789” for your reference)

# cat /etc/wpa_supplicant.conf

ctrl_interface=/var/run/wpa_supplicant

ctrl_interface_group=0

update_config=1

network={

ssid="TEST"

psk="123456789"

}

# wpa_supplicant -i wlan0 -c /etc/wpa_supplicant.conf -B

# dhclient wlan0

- If AP is configured with internet connection, user can use ping to check the internet availability as follows.

  - Check network connection after obtained IP address

# ping 8.8.8.8

Test Bluetooth

- Bring up Bluetooth Interface

- Bring up Bluetooth driver with follows if have mount USB stick underlying /run/media/sda1 on RSB-4810

# cp /run/media/sda1/rtl8852cu_* /lib/firmware/

# cp /run/media/sda1/rtk_btusb.ko /tmp/

# sudo insmod /tmp/rtk_btusb.ko

Note: you will see the interface hci0 by “hciconfig –a” command

- Test Bluetooth Interface

- Using Bluetooth with following commands

# hciconfig hci0 up

# bluetoothctl

[bluetooth]# scan on

[bluetooth]# scan off

If there are any Bluetooth devices nearby you will see some devices listed by “devices” command as follows.

[bluetooth]# devices

How To Build Driver with RSB-4810 BSP

- Download RSB-4810 BSP with following URL:

http://ess-wiki.advantech.com.tw/view/Linux_BSP_User_Guide_for_rk3568_series_Debian10

- Checkout RK3568_RISC_DIV130035 source code

# cd kernel

# git checkout rk3568_RISC_DIV130035

# make ARCH=arm64 rk3568_adv_defconfig

# make ARCH=arm64 rk3568-rsb4810-a2.img

- Copy driver zipped source file (RTL8852CE_WiFi_linux_v1.19.4.5-0-g285f1de55.20231027_RSB4810.zip) to your BSP build host with USB stick (assumed USB stick is mounted on /run/media/sda1/)

# cp /run/media/sda1/RTL8852CE_WiFi_linux_v1.19.4.5-0-g285f1de55.20231027(131786)_RSB4810.zip /tmp/

- To build Wi-Fi driver (rtkm.ko, 8852ce.ko) as following steps:

# cd RTL8852CE_WiFi_linux_v1.19.4.5-0-g285f1de55.20230127

# sudo make ARCH=arm64 –C /home/adv/BSP/rk3568_linux_risc/kernel M=`pwd` modules

- To build Bluetooth driver (rtk_btusb.ko) as following steps:

# cd 20230413_LINUX_BT_DRIVER_RTL8852C_COEX_v0707/

# sudo make INTERFACE=usb

Notes: Bluetooth firmware (rtl8852cu_fw&rtl8852_config) is available at “rtkbt-firmware/lib/firmware/”