How to Setup Network with AIW-357

on Yocto

Prerequisite

- ARK-1221 and 64G SSD storage

- Prebuild Yocto Image “ark-1221-yocto.img.bz2”

- Tarball file “aiw-357-mbim-set-ip.tar.bz2”

- Tarball file aiw357_yocto-linux.tar.bz2 if needed to porting driver

Flash image to SSD

We flash the prebuild Yocto image to SSD by the below command on Ubuntu 20.04.

# bzcat ark-1221-yocto.img.bz2 | sudo dd of=/dev/sdX

Note: /dev/sdX is depend on your system to find out the SSD storage

Booting on ARK-1221

After flash image complete, please select the item “bzImage-my” in grub menu as below screenshot and then the Yocto Linux can be booted by SSD with ARK-1220.

The user can login with default user “root” without password.

Network Test

Please follow the below steps to test network on Yocto Linux.

- Check some files that are build-in at home directory “/home/root on Yocto Linux.

- aiw-357-mbim-set-ip.tar.bz2 (application to bring up the network interface)

- aiw357_driver/mtk_pcie_wwan_m80.ko (AIW-357 driver)

- Check AIW-357 module with PCIe

# lspci

- Load driver

# insmod aiw357_driver/mtk_pcie_wwan_m80.ko

If the driver loaded successful, the user can find out device nodes as below.

- Prepare some files for MBIM

  - Decompress the tarball aiw-357-mbim-set-ip.tar.bz2” and you will see the script “mbim-set-ip”

# tar jxvf “aiw-357-mbim-set-ip.tar.bz2”

  - Create the mbim configuration and copy it to /etc directory

# vi /etc/mbim-network.conf

Please fill your APN in the below orange rectangle

- Turn radio on

# mbimcli -d /dev/ttyCMBIM0 -p --set-radio-state=on

Note that please wait for 1-2 minutes to let module attaching to base station after running this command.

- Start network connection

# mbim-network /dev/ttyCMBIM0 start

- Set IP and related setting with connection

# ./mbim-set-ip /dev/ttyCMBIM0 ccmni0

- Ping test

# ping -c 3 -I ccmni0 8.8.8.8

AT command

We can use AT command to communicate with AIW-357 module.

- Check device node ttyC0 whether exist or not

# ls /dev/ttyC*

- Get into AT command console by “Minicom”

  - Start to run minicom and select the “Serial port setup”

# minicom –s

  - Please set serial setting as below screenshot.

  - Test by AT command “AT” or “ATI”

How to Porting Driver to Yocto with ARK-1221

We setup development environment on Yocto Linux with ARK-1221. Please refer to the below steps for porting AIW-357 driver to Yocto Linux. The below table is about what we need on Yocto Linux after the tarball “aiw357_yocto-linux.tar.bz2” is extracted

| Directory | Description |
| --- | --- |
| aiw357_driver | AIW-357 driver source code |
| kernel | Yocto kernel source code |
| packages | Packages needed to install on Yocto |
| rootfs | The file of root file system is needed when building kernel |

- Prepare some packages that are installed on Yocto Linux.

  - packages/bison-3.8.2-r0.corei7_64.rpm

  - packages/gcc-plugins-11.3.0-r0.corei7_64.rpm

  - packages/libmbim-1.26.2-r0.corei7_64.rpm

  - packages/libmbim-bash-completion-1.26.2-r0.corei7_64.rpm

  - packages/lrzsz-0.12.20-r6.corei7_64.rpm

  - packages/minicom-2.8-r0.corei7_64.rpm

- Prepare the kernel source.

- kernel/linux-5.15.49.tar.bz2 (kernel source)

- kernel/boot_yocto.tar.bz2 -> boot_yocto/config-5.15.49-rt47-intel-ese-standard-lts (kernel configuration)

Please make sure the kernel setting with the below options if the user has the customized configuration：

  - Enable PCIE related options

    - CONFIG_PCIEPORTBUS=y

    - CONFIG_PCIEAER=y

    - CONFIG_PCIEASPM=y

    - CONFIG_PCIEASPM_DEFAULT=y

    - CONFIG_PCI_MSI=y

  - Disable MTK T7XX

    - CONFIG_MTK_T7XX is not set

  - Locate the file system file if needed

    - CONFIG_INITRAMFS_SOURCE="/home/root/rootfs/mender-initramfs-intel-corei7-64.cpio"

- Rebuild the kernel and modules

- Build the kernel

# make bzImage

- Build the modules and install it to root file-system

# make modules

# make modules_install

- Copy the kernel image and related files to /boot directory.

# cp /kernel_src/arch/x86/boot/bzImage /boot/bzImage-5.15.49-rt47-intel-ese-standard-lts

# cp /kernel_src/System.map  /boot/System.map-5.15.49-rt47-intel-ese-standard-lts

# cp /kernel_src/Module.symvers  /boot/Module.symvers-5.15.49-rt47-intel-ese-standard-lts

# cp /kernel_src/.config  /boot/config-5.15.49-rt47-intel-ese-standard-lts

- Create soft link to kernel image

# cd /boot

# ln –sf bzImage-5.15.49-rt47-intel-ese-standard-lts bzImage-my

- Reboot by the rebuild kernel

- Modify boot-loader to boot from rebuild kernel.

In Yocto Linux, the boot configuration is at /boot/efi/EFI/BOOT/grub.cfg . Please refer to “kernel/boot_yocto.tar.bz2 -> boot_yocto/grub.cfg” for the below screenshot.

- Build the AIW-357 driver

- Decompress the driver tarball “aiw357_driver/xs5g03_pcie_driver_Advantech.tar.gz”

# tar zxvf xs5g03_pcie_driver_Advantech.tar.gz

- Build the driver

# cd XS5G03_pcie_driver_Advantech

# make

If successful, you will find out the driver “mtk_pcie_wwan_m80.ko”