EWM FAQ

| Category | EWM | Date | 2018/09/26 |
| --- | --- | --- | --- |
| Keyword | EWM-C109F601E, EWM-C117FL0XE,3G/4G Connection,PPP mode,Ubuntu 16.04 LTS and above version |  |  |

- Question:

How to establish 3G/4G connection with EWM-C109F601E/ EWM-C117FL0XE automatically when system startup on Ubuntu Linux OS?

- Instructions :

Step 1

Install “Wvdial” application by Typing “apt-get install wvdial” command in terminal

Step 2

Create your “wvdial.conf” profile in specific path (/etc/wvdial.conf) which must refer to your sim card information

<Example_3G>

[Dialer Defaults]

Baud = 115200

Modem = /dev/ttyACM0                //select correct ACM port

Init1 =AT+CGDCONT=1,"ip","internet"

Phone=*99#

Username=’’    //please enter Apostrophe keys twice when the username is empty

Password=’’    //please enter Apostrophe keys twice when the password is empty

Dial Command =ATDT

NEW PPPD=yes

NOTE: Please remember to modify your APN(internet) and the serial port number of Modem before saving profile

<Example_4G>

[Dialer Defaults]

Baud = 115200

Modem = /dev/ttyACM0           //select correct ACM port

Init1 =AT+UCGDFLT=1,"ip","internet"

Phone=*99***4#

Username=’’  //please enter Apostrophe keys twice when the username is empty

Password=’’  //please enter Apostrophe keys twice when the password is empty

Dial Command =ATDT

NEW PPPD=yes

NOTE: Please remember to modify your APN(internet) and the serial port number of Modem before saving profile

Step 3

Input SIM card’s PIN code in “minicom” application before 3G connecting

Step 4

Exit “minicom” application (CTRL+A+Z) and input “wvdial &” command to establish 3G connection

Step 5

After connecting 3G/4G by wvdial application, please cross check if the ppp interface has generated in network interface

Step 6

Create a shell script to execute “wvdial” application under specific folder(/home/Desktop/Startup) <refer to attached script file below>

Step 7

Create a startup service under specific folder(/etc/systemd/system/)which can execute “autoDialup_script.sh” script when boots up system automatically <please refer to attached service file below>

<Directory modification>

/etc/systemd/system/AutoDialUp.service

[Unit] Description=Auto Dial up service After=network.target systemd-sysctl.service systemd-modules-load.service [Service] User=root WorkingDirectory=/home/advantech/Desktop/Startup Restart=no Type=oneshot ExecStart=//home/advantech/Desktop/Startup/autoDialup_script.sh RemainAfterExit=true [Install] WantedBy=multi-user.target

Step 8

Double check if the service has created under specific folder

Step 9

Shut down system and confirm if the 3G/4G connection can be established after booting up